Preservation of Bridge Deck – Peer Exchange Notes 9/11/17

NJ Dot using corrosion inhibitor (federally funded)

NJ/VT/NH Overlays state funded, when used, asphaltic, otherwise, bare deck

LMC & Epoxy overlays

ME/PA using membranes.  8-12 years mill & fill when cracking occurs to extend the life of overlay

NH, ME & VT washing annually.  PA seeping and flushing annually.

PennDOT looking for stats on bridge washing effectiveness.

DE does not wash decks.

Surface preparation preferences: PennDOT saw-cut/jack hammer for patching.  Hydro demo no larger projects PA & NYS.

Deck patch material preferences: PennDOT PCC, rapid set.  Depends on time restrictions.  NYS uses PCC when time allows.  ME using a four-hour concrete (designed in-house). VT PCC, others if RE allows.  DE HES & Rapid set.

Scoping deck preservation project: NYS using GPR, VDOT Chloride, PennDOT sounding decks.