California

DOT Research Page     http://www.dot.ca.gov/research/

2014 Annual Report     http://www.dot.ca.gov/research/docs/DRISI_AR2013_14.pdf

Projects

| Deck | 2346 Controlling Temperature and Shrinkage Cracks in Bridge Decks and Slabs | Lee, Peter 05/01/16 |
| --- | --- | --- |
| Deck | 2543 Validation of Polyester Concrete Rehabilitation Strategy to Extend the Service Life of Concrete Bridge Decks | Sahs, Steve 06/30/15 - |
| Scour | 2403 Bridge Pier Scour Research, TPF-5(211) | Flora, Kevin 12/31/14 |
| Inspection | 1050 Underwater Inspection of Bridge Substructures Using Underwater Imaging Technology, TPF-5(131) | Johnson, Michael 06/30/15 – |
| Monitoring | Evaluation of the VTI ECI-1 Corrosion Monitoring Device, link |  |
| Monitoring | Passive Embedded Chloride Threshold Sensors, link |  |
| Deck | Investigation of Methacrylate Rehabilitation Strategy to Extend the Service Life of Concrete Bridge Decks, link | Eli Cuelho and Jerry Stephens - Box girder design. - Deck is integrally cast as part of the load bearing structure. - Investigate the effectiveness of high molecular weight methacrylate (HMWM) treatments applied at various stages of bridge deck deterioration - Eight full-scale deck panels (four sets of two panels each) were cast to Caltrans specifications and fixed to a support structure to simulate a box girder design configuration. - A fully-automated, 20-kip, rolling wheel load simulator was used to traffic the test panels. - Six of the eight test panels subsequently were treated with HMWM at various traffic levels, (up to 2,000,000 cycles at various cycle increments), - Four treatment/traffic combinations including untreated (control) samples. - Evaluation on cracking behavior and flexural stiffness of the test panels over time. - This result was most evident in the two sets of deck panels that were treated at 1,000,000 load cycles and then trafficked for an additional million load Cycles - Test panels treated at earlier times also showed a benefit, but to a much lesser extent. - Based solely on traffic-induced stresses and not environmental induced deterioration, performance comparisons generally indicated that later applications of HMWM will likely result in greater structural benefit. |