Midwest Bridge Meeting Deadwood, SD October 18, 2023

Session 5.2 Peer Exchange Load Rating and Posting

When to load ratings need to be updated:

Inspector recommendation, deterioration or condition changes, damage, new required legal vehicles, different analysis method, load testing, new bridges, material testing, NDE.

What rating vehicles does your state require?

Local special vehicles, routine permit vehicles, timber trucks, coal trucks, added weight allowance for natural gas-powered vehicles.

Ways to get Permit loads across bridges:

Single vehicle, reduced lanes, modify travel lanes away from damaged girders with traffic control.

Reduce speed, remove all or some of any wearing surface when gravel, asphalt, etc.

Load rating preservation alternatives

Slow deterioration by limiting chlorides,

Restrict or minimize overloads allowed,

Reduce ADT/ADTT – alternate routes,

Reduce speed limit – add lights – should reduce impact

Load Posting signs vary between states. Signs might include # of axles, SHV’s, EV’s, gross weight, etc.